TASK 1 SOLUTIONS:

I used a photo resistor in series with a normal resistance and then ran a test ckt in which I measured the potential drop a/c the normal resistor as I went from min possible value of light to maximum possible value of light(the values were 6 and 679). Then I used basic if else statements to turn the led on after a certain threshold.

Ckt:

int voltpin=A2;

float volt;

int ledpin=8;

void setup()

{

pinMode(voltpin,INPUT);

Serial.begin(9600);

pinMode(ledpin,OUTPUT);

}

void loop()

{

float val=analogRead(voltpin);

volt=(val/1023)*5;

Serial.print(volt);

Serial.print("   ");

Serial.println(val);

if(val<400){

digitalWrite(ledpin,HIGH);

}

else{

digitalWrite(ledpin,LOW);

}

delay(100);

}

https://www.tinkercad.com/things/jvwpp2wA0c3-envision-task1q1

TASK 2 SOLUTION:

I used the adafruit github repo for the functions.

#include <Adafruit_LiquidCrystal.h>

Adafruit_LiquidCrystal lcd(0);

int pin1=9;

int pin2=7;

int pin3=5;

int pin4=3;

int num1,num2;

void setup() {

Serial.begin(9600);

lcd.begin(16, 2);

lcd.setBacklight(HIGH);

lcd.print("hello, world!");

delay(2000);

lcd.clear();

pinMode(pin1, OUTPUT);

pinMode(pin2, OUTPUT);

pinMode(pin3, OUTPUT);

pinMode(pin4, OUTPUT);

}

void loop() {

Serial.println("Enter your number1: ");

while(Serial.available()==0){

}

num1=Serial.parseInt();

Serial.println(num1);

Serial.println("Enter your number2: ");

while(Serial.available()==0){

}

num2=Serial.parseInt();

Serial.println(num2);

int stat1,stat2,stat3,stat4;

int num3=num1+num2;

if(num3>15){

lcd.print("OVERFLOW!!");

delay(4000);

}

else{

stat4=num3/8;

int rem1=num3%8;

stat3=rem1/4;

int rem2=rem1%4;

stat2=rem2/2;

int rem3=rem2%2;

stat1=rem3;

if(stat1==1){

digitalWrite(pin1,HIGH);

}

else{

digitalWrite(pin1,LOW);

}

if(stat2==1){

digitalWrite(pin2,HIGH);

}

else{

digitalWrite(pin2,LOW);

}

if(stat3==1){

digitalWrite(pin3,HIGH);

}

else{

digitalWrite(pin3,LOW);

}

if(stat4==1){

digitalWrite(pin4,HIGH);

}

else{

digitalWrite(pin4,LOW);

}

lcd.print("SUM IS :");

lcd.setCursor(0,1);

lcd.print(num3);

delay(5000);

lcd.clear();

}

lcd.clear();

digitalWrite(pin1,LOW);

digitalWrite(pin2,LOW);

digitalWrite(pin3,LOW);

digitalWrite(pin4,LOW);

}

I first initialize the lcd display by setting the backlight to HIGH, and then I display hello world to check if it works(not necessary). And then initialize the pin modes. In the loop function, I take the input from the user with the help of serial monitor using the serial.parseInt function. The while(serial.available==0) statement makes the serial monitor wait till the user inputs his number t=by pressing enter. I take two inputs, store them in two variabes, and then add them to store in a third variable. Then I check if its more than 15: if yes, then I display overflow message, wait for 4 secs, and then repeat the process…if no, then I divide the num3 by 8, as its an division of integers,the output will be integer( and it can be 0 or 1(0 if number is less than 8, 1 if sum is greater than 8), then store the remainder in another variable, and then repeat the process for 4 and 2. Then I store the results in 4 vribles which will then determine the state of pin(HIGH is 1, LOW if 0)(I used basic if/else for this part of program). Then once the output is displayed, I clear the lcd, and the loop function begins again.

https://www.tinkercad.com/things/jvwpp2wA0c3-envision-task1q1